EXPERIMENT 2 – STUDY OF NETWORK CABLES

# AIM

To study different types of network cables and to make a straight-through cable and crossover cable using an RJ45 connector and crimping tool.

# REQUIREMENTS

- Ethernet cable (CAT5/CAT5e)

- RJ45 connectors

- Crimping tool

- Cable tester (if available)

# TYPES OF NETWORK CABLES

- UTP – Unshielded Twisted Pair

- STP – Shielded Twisted Pair

- Coaxial Cable

- Fibre Optic Cable

# PROCEDURE

- Take an Ethernet cable and cut it to the required length.

- Strip approximately 1.5 cm of the outer cable covering.

- Untwist and separate the four pairs of wires.

- Arrange the wires according to the required wiring standard.

- For a straight-through cable, use the same arrangement at both ends.

- For a cross-over cable, use arrangement A at one end and arrangement B at the other end.

- Cut the wires evenly and insert them into the RJ45 connector.

- Ensure that all wires reach the end of the connector in the correct order.

- Insert the RJ45 connector into the crimping tool and press firmly.

- Repeat the process for the other end.

- Test the completed cable by connecting it to the appropriate devices.

# OBSERVATION

| Cable | Wiring Arrangement | Usage |
| --- | --- | --- |
| Straight-through cable | Same standard at both ends | PC to Switch/Router |
| Cable | Wiring Arrangement | Usage |
| Cross-over cable | Different standards at each end PC to PC |  |

# RESULT

Thus, the different types of network cables were studied, and straight-through and crossover Ethernet cables were successfully prepared using RJ45 connectors and a crimping tool. The cables were tested and found to be working properly.